Adaptive Navigation Robot Using AI-Based Dynamic Path Planning in a Simulated Environment

Team Members:
Luca Bugeja, Luigi Camilleri, and Matthias Ellul

Project Overview:
This project aims to design and implement an intelligent mobile robot capable of autonomously navigating towards a target in a dynamic simulated environment while avoiding both static and moving obstacles. The system will perceive its surroundings through simulated sensors, reason about potential collisions, and act by adjusting its trajectory in real time. The robot will integrate perception, planning, and control modules using the ROS 2 framework, while the world simulation will be generated using Gazebo.

Objectives:

- Enable autonomous navigation in a dynamic 2D environment.

- Detect and track obstacles using simulated LIDAR and camera data.

- Plan and replan safe paths in response to environmental changes.

- Evaluate the system’s performance using metrics such as path efficiency, success rate, and average time to goal.

- Demonstrate an AI reasoning layer that enables adaptive decision-making.

System Components:

- Perception: Uses simulated sensors (LIDAR, camera) to detect obstacles and estimate their motion, while also allowing the robot to sense its surroundings and internally build the map’s layout in real time.

- Localization & Mapping: Employs SLAM (via slam_toolbox) to learn the simulated environment dynamically and maintain a live occupancy grid map.

- Planning:

  - Global planner: A* on the occupancy grid.

  - Local planner: Dynamic Window Approach (DWA) or TEB for obstacle avoidance.

- Decision Layer: Behavior Tree or FSM to manage navigation states (navigate, avoid, replan, recover).

- Control: Differential drive velocity controller for smooth motion.

Tools and Frameworks:
ROS 2 (Jazzy), Gazebo Fortress, Nav2 Stack, SLAM Toolbox, OpenCV, Python/C++ nodes.

Evaluation Metrics:

- Success rate of reaching goal without collision.

- Path efficiency (% deviation from optimal path).

- Average time-to-goal.

- Number of dynamic replans per trial.

- CPU utilization (for efficiency analysis).

Expected Deliverables:

- ROS 2 simulation package with launch files and source code.

- Live demonstration of the simulation.

- 5-page technical report and GitHub repository containing all project code.